PPDint01mi!

# Document Revision Control

| Revision Number: | Date: | Key Points |
| --- | --- | --- |
| A | 24/07/2024 | First Issue |

# Key Information

## Description:

Saint Gobain requires that a consistent equipment identification system is used for the entirety of a production facility, and in addition maintain a view that the same identification system is adopted throughout new and existing Saint Gobain sites.

This standard describes the preferred identification system that should be used, with the intent to produce an cascading identification system from the site level through each individual production line and machine to uniquely identify every motor, instrument and wear component on each site.

## Affected Areas:

To maintain a consistent equipment identification system across all aspects of plant operations, Saint Gobain requires that the defined identification system be used in all of the following areas:

Identification of all mechanical equipment on:

- Engineering Line Drawings (ELDs)

- Piping and Instrumentation Diagrams (P&IDs)

- Process Flow Diagrams (PFDs)

- General arrangement drawings

- Plant Layout Drawings

- Equipment schedules

- Labels and Identification Plates (Including CE Marking)

The Equipment Identification must be uniform across the following areas:

- Electrical panel and cable references

- Control software references

- Instruction Manual references

- Maintenance Information references

- Recommended Spares Parts Lists

- Pipelines (Process & Services)

## Extents of Identification System:

The Saint Gobain identification system will encompass the following equipment:

- Machines (or equipment assemblies)

- Drive motors

- Gearboxes

- Wear components

- Actuated valves (including air solenoid valves)

- Air cylinders

- Instruments

- Pipelines

## Identification Format:

The identification system comprises of a series of separate fields conjoined together to provide a unique reference to each item of equipment on a production site as well as some descriptive information to assist in recognising the type of equipment drawn just from the Identifier.

It is a requirement that all equipment and assemblies must be uniquely identified using Characters 1-8 across the production plant, with further characters (13-17) used to identify instrumentation and equipment that is mounted on the same machine.

It is important to note that identifiers for piping/ducting are somewhat different to the identification format detailed below. The first 8 characters (3.4.1-3.4.5) are unified with the Identifier system, ensuring that equipment and piping are given unique identifier codes for the purpose of system referencing, but the characters after this (11-17) follow a different identifier coding. This is detailed in Section 3.5 to avoid confusion.

The delimiter between fields of the identifier must be an underscore “_”, unless a particular computer system or maintenance management program does not allow this character. In this case an alternative “-“or “/” can be used.

Unified Identification Format:

For the OTO Quinto Filter Belt system the meaning of each character in the identifier is detailed below:

### Line Number (character 1 - letter)

Saint Gobain production sites can have several different production lines including plasterboard lines, bag plaster lines, cove lines, secondary manufacturing lines, etc. As the OTO Process equipment is common to multiple lines then all items are denoted under the letter Z for Common line.

### Process Area (character 2 - letter)

The production process is divided into Process Areas designated by a letter. For the Filter Belt Equipment this is designated as C for Stucco Conditioning.

### Process Step (characters 3-4 - numbers)

Each process is sub-divided into process steps designated by 2 numbers.

For the Filter Belt Assembly and the Wash Water components, this is 50 for Slurry Dewatering.

For the Vacuum Pump Skid and other components, this is 51 for Vacuum System.

### Separator (character 5 - Delimiter)

Field delimiter, normally an underscore, ‘_’ or similar character.

### Equipment Item (characters 6-8 - numbers)

This 3 number code identifies the machine assembly in the process step. The Equipment Item numbers must be unique for each item within a process step (For Example, there should only be one AE18_010 notated equipment), but the numbering can be repeated in other process steps (e.g., AE18_010, AF21_010).

It is important to note that in cases where instruments or equipment items are directly linked to a main assembly item or functionally considered embedded in a larger equipment piece then these items will bear the same identifier as the larger equipment piece. These are then uniquely identified using characters 9-17.

For the Filter Belt Assembly the equipment numbers 001-100 have been allocated

For the Wash Water components, the equipment numbers 101-200 have been allocated

For the Vacuum Pump Skid and other components, the equipment numbers 001-200 have been allocated

Following this methodology, characters 1 through to 8 will provide a unique identifier for any machine on site.

### Identifier Summary (characters 1-8 Combined)

In summary of the equipment identifier section of the equipment numbering:

For the Filter Belt Assembly, each equipment should read ZC50_XXX with XXX providing unique identifiers from 001-100.

For the Wash Water Components, each equipment should read ZC50_XXX with XXX providing unique identifiers from 101-200.

For the Vacuum Pump Skid and other components, each equipment should read ZC51_XXX with XXX providing unique identifiers from 001-200.

### Equipment description (characters 9-11 - letters)

These 3 characters provide a description of the generic type of machine or valve as listed in Chapter 4.

This descriptor may be useful in searching for similar equipment with a maintenance or inventory list and for easy identification by Operators/Engineers.

For Instrumentation that is not linked to a particular machine (e.g. instrument in piping between multiple machines), to maintain the progressive listing of identifiers the equipment description characters for such instrumentation will be labelled ‘INS’. Section 4.3 onwards will remain the same for these instruments.

In the case of piping, these characters are used to detail what material is contained within the pipe, with a list of materials given in Section 4.5.

### Separator (character 12 - Delimiter)

Field delimiter, normally an underscore, ‘_’ or similar character.

### Sub-assembly Item (characters 13-17 – letters & numbers)

This section of the identifier denotes sub-components of the machine (motors, gearboxes, bearings, etc.), or instruments that are attached to the machine. This field can be a mixture of letters and numbers and can be of variable length up to 5 characters.

This is primarily for equipment items that are assembled

#### Sub-assembly Components & Instrumentation (13-16)

Characters 13-15 define the type of instrument or sub-assembly component. A list of codes is given in Chapters Chapter 4 Sections 1 & 2, Chapter 12 focussing on instruments and Chapter 13 focussing on components.

It is possible several different types of instruments and a number of instruments of the same type are all part of one machine.

If there are several instruments of the same type on a single machine, then there needs to be a number included for unique identification (“TT01” and “TT02”).

For instruments of different types the identifying numbers can all start from 1. So “TT01” and “PT01” may exist together on the same piece of equipment, but the complete identifier is unique.

For the purpose of equipment identification, a position sensor (“ZS01” or “ZT01”) will denote the position or presence of a part of a machine. The position or presence of a process consumable or product (e.g., paper or plasterboard) will be measured by flow sensors (“FS01” or “FT01”).

To differentiate positions of valves and related equipment the position identifier may carry an extra letter to define an open or closed position – “ZSO” or “ZSC” followed by a number if there is more than one switch on the valve. Similarly, the identifier can be extended using “H” for high, “L” for low, “LL” for low low, etc.

#### Sub-assembly Component Numbering (14-16)

Following the component and instrumentation will be a number, which identifies the individual component from similar components on the same machine. Where only 1 motor exists, this will be identified as “M01”. Where several motors exist on the same machine these will be identified as “M01”, “M02”, etc.

The associated gearboxes will be identified as “GX01”, “GX02”, etc to match the motor to which they are attached. Each motor may drive a shaft with bearings attached. The bearings will then be identified as “BR11” and “BR12” for the first and second bearing associated with motor “M01”. A similar system should be adopted for other components with association to drives.

Instruments associated with a machine will be identified in this field using the descriptors given in Chapter 12 and a number, which individually identifies it from other similar instruments on the machine. For example, a machine may have temperature transmitters “TT01”, “TT02”, etc. and position switch “ZS01” and “ZS02”.

Where instruments are associated with a particular drive motor on the machine (encoder, position switch, etc.), then the identification number should maintain a link with the motor number as described for the gearboxes above.

Important: Exceptions may need to be made where there are more than 9 motors on a machine in which case motor 1 may need to be identified as “M01” and the associated bearings as “BR011” and BR012”, etc. Similar modifications to this level of the identification may be necessary depending on the details of the machine and the number of parts. In general, it is advised to split such a complex machine down and identify sections of it as different identifier items to make it easier to name and comprehend the system.

### Further identifiers

Additional characters can be added to the identifier for specific uses. The definition of these characters are outside of the scope of this specification and must be agreed and notarised on a project to project basis.

Examples of cases where this might occur are:

- MCC panel components related to a particular motor.

- PLC software inputs and outputs to motors, instruments etc.

- Motor fault conditions identified within the plc software.

## Piping Identifier Format:

The pipeline codes shall now follow the same format as that for the rest of the equipment identification:

AE10_026PRW_075S1

Characters 1-8 will uniquely identify the pipeline.

Characters 9-11 will identify the fluid to be carried, as defined in Section 4.5.

Characters 13-15 will identify the pipeline diameter (nominal bore).

Characters 16 & 17 will identify the pipeline material as defined in Section 4.6.

# Equipment Identifier Usage

It is understood that in certain circumstances the use of the full equipment identifier can lead to problems with too many characters. In these cases, it will be acceptable to drop certain parts of the identifier in order to operate with a more sensible number of characters.

## P&IDs, ELDs and PFDs

In the case of system drawings (P&IDs, ELDs and PFDS), the line number and process area code (characters 1-5) may be covered by a general note on each drawing and need not be shown for each item that is shown in the drawing. It is important to note that this is only acceptable when the line number and process area code are identical for all items on the related drawing, this method should not be used if there is the possibility of confusion or mislabelling.

## Control Software

For control software it may be desirable to drop the equipment description (characters 9-11). Additionally, if the PLC in which the software resides is linked solely to a single production line and process area then characters 1 and 2 of the identifier can be removed from the identifiers.

Other situations need to be considered on their own merits, but the ideal will be to maintain unique identification of the component in a form that anyone aware of this identification methodology.

# Identifier Lists

## Equipment descriptor

The following codes are specified as equipment descriptors :-

ACU		Air conditioning unit

AGI		Agitator

ASD		Air separator – dynamic

ASS		Air separator – static

BBH		Bulk bag handler (discharger)

BFT		Board forming table

BNR		Burner

BRU		Brush / cleaner

BSB		Board smoothing bar

BSP		Board spray pipe

BWB		Board walker wheel

CMP		Compressor / blower

COA		Conveyor – air slide

COB		Conveyor – belt type

COC		Conveyor – chain type

COP		Conveying – pneumatic

COR		Conveyor – roller type

COS		Conveyor – screw type

CPA		Compactor

CRS		Creasing / scoring system

CRU		Crusher

CUT		Cutter

CYC		Cyclone dust separator

DBX		Disengagement box

DCL		Dust collecter

DPG		Dump gate

DPT		Dip tube

DRY		Dryer

EDG		Edge guide system

ELV		Elevator

ENC		Enclosure / fence

EXP		Expander / Exfoliator

EXR		Explosion relief system

EXT		Extruder

FAN		Fan

FAR		Flame arrestor

FDR		Feeder

FLX		Flexible Connection

FRD		Filter / regulator / drain – compressed air

FRL		Filter / regulator / lubricator – compressed air

FTR		Filter / Strainer

FUS		Fusible link

GLU		Glue system

HBR		High burn reactor

HEX		Heat exchanger

HTP		Hot pit

HTR		Heater – inline or immersion

IBC		Intermediate bulk container (semi bulk)

IGN		Ignition system

INS		Stand-alone instrument

INV		Inverter

KET		Kettle

KNO		Knocker / vibrator

LFT		Lifting equipment

LUB		Lubrication unit

MCC		Motor control centre

MDR		Metal detector / remover / magnet

MIL		Mill

MIX		Mixer

PAK		Packer

PAL		Palletiser

PAN		Electrical panel

PHD		Paper Hole Detector

PMP		Pump

PRT		Printing system

PSP		Pipeline special (nozzle, spool, etc.)

PUS		Paper unwind stand

RAD		Refrigerant air dryer

RAL		Rotary air lock

ROB		Robot (handling equipment)

ROL		Roller coater

SAM		Sampling system

SAW		Saw station

SCL		Stucco cooler

SCN		Screen

SDU		Silo discharger unit

SIL		Silo / hopper / hot pit / surge bin

SKT		Sack tip unit

SLN		Silencer

SPL		Splicer - paper

STB		Stop bar / Alignment device / Pusher

STR		Strapper

TAP		Tape application machine

TEN		Tension compensator - paper

TLD		Tanker loading device

TNK		Tank / sump

TRN		Turning mechanism / device

TTU		Truck tipper/tipping unit

VIB		Vibrator

VSL		Vessel – pressurised

WBF		Weigh belt feeder

WEH		Weigher – batch

WRP		Wrapping equipment

## Component descriptors

The following codes are specified as machine component descriptors:-

ACY		Air cylinder

BLT		Belt (conveyor belt)

BCN		Beacon

BR		Bearing

BRK		Brake

CLU		Clutch

CPL		Coupling

DR		Door (dryer for example)

DS		Disconnect switch / local isolator

FX		Flexible connection - pipeline

GX		Gearbox

H		Hoist

KLX		Klaxon

KY		Key / lock

LBM		Lifting beam

LC		Local controller

M		Motor

MS		Manual switch

PB		Push button

PL		Platform

PN		Pipeline special (spray nozzle)

R		Roller

SL		Seal

SOV		Solenoid valve

SP		Sample / measuring point

TX		Transformer

## Instrument descriptors

The following codes are specified as instrument descriptors:-

Note- all descriptors may be extended by the addition of L – Low, H – High, LL – Low Low, O – Open, C – Closed, etc.

AL		Alarm

BE		Burner flame scanner

BS		Burner switch

DE		Dust detection element / Burst bag detector

DT		Dust concentration transmitter (continuous monitoring)

EM		Emission monitor

ET		Electrical current transmitter

ES		Emergency stop or pull wire (induction loop, light barrier or gate switch)

FI		Flow indicator

FS		Flow switch – product flow

FT		Flow transmitter – product flow

GT		Gas detection transmitter

HT		Humidity transmitter

LI		Level indicator

LS		Level switch

LT		Level transmitter

MT		Moisture transmitter

ORP		Orifice plate

PDS		Differential pressure switch

PDT		Differential pressure transmitter

PI		Pressure indicator

PS		Pressure switch

PT		Pressure transmitter

SE		Speed sensing element

SS		Speed switch / rotation sensor

ST		Speed transmitter / encoder

TE		Temperature sensing element / Thermistor (hard wired)

TI		Temperature indicator

TS		Temperature switch

TT		Temperature transmitter

VEN		Venturi

WE		Weighing element (load cell)

WT		Weight transmitter

XE		Vibration sensing element

XT		Vibration transmitter

ZS		Position / proximity switch

ZSO		Position switch open

ZSC		Position switch closed

ZT		Position transmitter

## Valve descriptors

The following codes are specified as valve descriptors :-

VAA		Valve position controlled actuated

VAO		Valve on/off actuated

VAM		Valve manually operated

VAN		Valve none return

VAP		Valve pressure relief

VAR		Valve pressure regulation

VFM		Valve flow adjustment manual (needle valve)

## Service descriptors

The following descriptors are applied to pipeline contents

ACC		Accelerator

AMB		Ambient air

BHW		Borehole Water

CAR		Compressed Air

CDA		Common dry additives

CEM		Cement

CLW		Cooling Water

CLY		Clay

CON		Condensate

DCW		Dam control water

DSG		Dry Desulphogypsum

DSC		Dry starch

EMW		Mixer Emergency Water

ESW		Edge Spray Water

FAG		Foaming Agent

FMW		Foam Water

FOM		Foam

GAS		Natural Gas

GLU		Glue

GUG		Guar gum

HFO		Heavy Fuel Oil

HXW		Heat Exchanger Water

LBC		Liquid Biocide

LCS		Liquid Coating Solution

LFO		Light Fuel Oil

LMS		Liquid Microsilica

LPG  		Liquid Petroleum Gas

LSC		Liquid Starch

LST		Limestone

MIN		Mineral gypsum

MNW		Mine Water

MSW		Mixer Seal Water

POL		Liquid Polymer

PRW		Process Water

PTS		Potassium sulphate

PTW		Potable Water

RCA		Roller coater additive

RET		Retarder

SLO		Silicon oil

SLU		Gypsum Slurry

TWA		Towns water

VNT		General tank vent

WAW		Waste water

WAX		Wax emulsion

WRA		Water reducing agent

## Pipeline Material Specification Codes

The following codes are specified for pipeline materials:-

M1		Mild Steel (pressurised)

M2		Mild Steel (non-pressurised)

NY		Nylon and Plastic

S1		Stainless Steel 304

S2		Stainless Steel 316